EEE241L Multisim Assignment

Electrical Circuits II Lab

| Name: | Farhana Tasnim | Student ID: | 2532289043 |
| --- | --- | --- | --- |
| Section: | 04 | Date: | 08/09/2026 |

# Objective:

To simulate inductive and capacitive load circuits in NI Multisim, measure peak voltages and time differences using an oscilloscope, and calculate load current, phase angle, and power drawn by the load.

# Circuit Schematics:

Figure 1: Inductive load in a series circuit                      Figure 2: Capacitive load in a series circuit

# Problem 1: Inductive Load in a Series Circuit

Simulate the inductive load circuit shown in Figure 1. Measure the load voltage, the voltage across Ri, and the required time differences using the oscilloscope. Then complete Table 1 and calculate the load current and power drawn by the load.

## Table 1: Inductive Load

| Reading | Parameter | Value |
| --- | --- | --- |
| 1 | V_Load (peak) | 4.58 V (peak) |
| 2 | Δt for load voltage | 140 µS |
| 3 | θ for load voltage | θ = (Δt ×f ×360) = (140 ×10-6 ×50 ×360) = 2.52º ( Leading ) |
| 4 | V_Ri (peak) | 440 mV (peak) |
| 5 | Δt for Ri voltage | 880 µS |
| 6 | θ for Ri voltage / current | θ = (Δt ×f ×360) = (880 ×10-6 ×50 ×360) =15.84° (Lagging) |
| 7 | I_s ∠ θ | Is =VRiRi=440∠-15.84º1000 = 0.44 mApk ∠-15.84 º |
| 8 | Power drawn by the load, P | P = (12 Ipk2 ×Rload )= 0.5×(0.44×10-3)2×104 = 0.968 mW |

### Calculation ( Inductive Load ):

Reading 1: Circuit with oscilloscope connection

Screenshots ( Inductive Load ):

Reading 2:  Δt for load voltage

Reading 5: Δt for Ri voltage

Reading 4: V_Ri (peak)

Reading 1: V_Load (peak)

# Problem 2: Capacitive Load in a Series Circuit

Simulate the capacitive load circuit shown in Figure 2. Measure the load voltage, the voltage across Ri, and the required time differences using the oscilloscope. Then complete Table 2 and calculate the load current and power drawn by the load.

## Table 2: Capacitive Load

| Reading | Parameter | Value |
| --- | --- | --- |
| 1 | V_Load (peak) | 4.57 V (peak) |
| 2 | Δt for load voltage | 80 µS |
| 3 | θ for load voltage | θ = (Δt ×f ×360) = (80×10-6 ×50 ×360) = 1.44º ( Lagging ) |
| 4 | V_Ri (peak) | 420 mV (peak) |
| 5 | Δt for Ri voltage | 1.5 mS |
| 6 | θ for Ri voltage / current | θ = (Δt ×f ×360) = (1.5 ×10-3 ×50 ×360) = 27º ( Leading ) |
| 7 | I_s ∠ θ | Is =VRiRi=420∠27º1000 = 0.42 mApk ∠27 º |
| 8 | Power drawn by the load, P | P = (12 Ipk2 ×Rload )= 0.5×(0.42×10-3)2 ×104 = 0.882 mW |

### Calculation ( Capacitive Load ):

### Screenshots (Capacitive Load):

Reading 1: Circuit with oscilloscope connection

Reading 2:  Δt for load voltage

Reading 1: V_Load (peak)

Reading 5: Δt for Ri voltage

Reading 4: V_Ri (peak)