EEE241L Multisim Assignment

Electrical Circuits II Lab

| Name: |  | Student ID: |  |
| --- | --- | --- | --- |
| Section: |  | Date: |  |

## Objective

To simulate inductive and capacitive load circuits in NI Multisim, measure peak voltages and time differences using an oscilloscope, and calculate load current, phase angle, and power drawn by the load.

## General Instructions

Recreate each circuit exactly as shown in the given schematic.

Use the AC source values shown in the circuit: 5 V peak, 50 Hz, 0 degree phase.

Use the oscilloscope to measure peak values and time differences. Attach the required screenshots as evidence for the readings.

Use proper units for every measured and calculated value.

Show the necessary calculation steps below each table.

## Circuit Schematics

| Figure 1: Inductive load in a series circuit | Figure 2: Capacitive load in a series circuit |
| --- | --- |

## Problem 1: Inductive Load in a Series Circuit

Simulate the inductive load circuit shown in Figure 1. Measure the load voltage, the voltage across Ri, and the required time differences using the oscilloscope. Then complete Table 1 and calculate the load current and power drawn by the load.

### Table 1: Inductive Load

| Reading | Parameter | Value |
| --- | --- | --- |
| 1 | V_Load (peak) |  |
| 2 | Δt for load voltage |  |
| 3 | θ for load voltage |  |
| 4 | V_Ri (peak) |  |
| 5 | Δt for Ri voltage |  |
| 6 | θ for Ri voltage / current |  |
| 7 | I_s ∠ θ |  |
| 8 | Power drawn by the load, P |  |

Required screenshots:

Circuit with oscilloscope connection for Reading 1

Zoomed oscilloscope screen for Reading 1

Zoomed oscilloscope screen for Reading 2

Zoomed oscilloscope screen for Reading 4

Zoomed oscilloscope screen for Reading 5

## Problem 2: Capacitive Load in a Series Circuit

Simulate the capacitive load circuit shown in Figure 2. Measure the load voltage, the voltage across Ri, and the required time differences using the oscilloscope. Then complete Table 2 and calculate the load current and power drawn by the load.

### Table 2: Capacitive Load

| Reading | Parameter | Value |
| --- | --- | --- |
| 1 | V_Load (peak) |  |
| 2 | Δt for load voltage |  |
| 3 | θ for load voltage |  |
| 4 | V_Ri (peak) |  |
| 5 | Δt for Ri voltage |  |
| 6 | θ for Ri voltage / current |  |
| 7 | I_s ∠ θ |  |
| 8 | Power drawn by the load, P |  |

Required screenshots:

Circuit with oscilloscope connection for Reading 1

Zoomed oscilloscope screen for Reading 1

Zoomed oscilloscope screen for Reading 2

Zoomed oscilloscope screen for Reading 4

Zoomed oscilloscope screen for Reading 5

## Screenshot Requirements

For each circuit, attach the following screenshots:

Full circuit showing the oscilloscope connection for Reading 1.

Zoomed oscilloscope screen for Reading 1.

Zoomed oscilloscope screen for Reading 2.

Zoomed oscilloscope screen for Reading 4.

Zoomed oscilloscope screen for Reading 5.

## Submission Instructions

Submit one PDF file through the assigned submission platform.

The file must include completed tables, units, and the required screenshots.

Name the submitted file using this format: StudentID_Name_EEE241L_Multisim_Assignment.

Unreadable screenshots or missing calculation steps may reduce marks.

## Mark Distribution

| Assessment item | Marks | Notes |
| --- | --- | --- |
| Problem 1: Inductive load readings, screenshots, and calculations | 8 | Table 1 and required screenshots |
| Problem 2: Capacitive load readings, screenshots, and calculations | 8 | Table 2 and required screenshots |
| Presentation, unit consistency, and final submission quality | 4 | Clean formatting and correct units |
| Total | 20 |  |